MINDORO STATE UNIVERSITY

Bongabong Campus

College of Computer Studies

Bachelor of Science in Information Technology

A.Y. 2025-2026

Second Semester

Propose Campus Network for Villapagasa National High School

Submitted to:

Ernesto B. Rodriguez Jr.

Instructor

As Course Requirements in ITP 311 – Networking 2

Submitted by:

Palma, Paolo Amor, – Team Leader

Rico, Angelica – Asst. Team Leader

Sevilla, Charles Kevin

Diaz, Jenny Rose

Almanza, Janwin

Magsino, Kyle Emanuel

Guyo, Charles Angel

Submition Date:

November 24, 2025

Chapter 1

INTRODUCTION

- Background

Villapagasa National High School, located in the municipality of Bansud, Oriental Mindoro, serves as a vital educational institution with 612 junior and 230 senior high school students, a total of 842 students, 44 teachers and 10 administrative staff members. As part of the Department of Education’s digital transformation initiative, the integration of information and communication technology (ICT) into daily academic and administrative operations has become essential for improving learning outcomes, streamlining processes, and ensuring equitable access to digital resources.

Currently, the school relies on a fragmented and outdated network infrastructure limited to isolated workstations in the administrative office and minimal connectivity in select classrooms. There is no centralized data management system, limited Wi-Fi coverage, and no formal network segmentation—leading to security vulnerabilities, inefficient resource sharing, and poor scalability.

To address these challenges, this project proposes the design and implementation of a modern, structured campus-wide network infrastructure based on the Cisco hierarchical model (Core, Distribution, Access layers). The new network will support e-learning platforms, digital classroom tools, centralized file and print services, secure internet access, and future-ready technologies such as IP telephony, smart boards, and cloud-based applications.

This project is developed as part of the course requirement for ITP 311 – Networking 2 at Mindoro State University – Bongabong Campus, under the Bachelor of Science in Information Technology program. It reflects both theoretical knowledge and practical application of networking concepts including VLAN segmentation, IP addressing, routing, switching, wireless deployment, and network security.

- Objectives

General Objective

To design a secure, scalable, and efficient campus network infrastructure for Villapagasa National High School that supports academic, administrative, and operational needs through centralized services and industry-standard networking practices.

Specific Objectives

- To implement VLAN segmentation to isolate traffic for Admin, Faculty, Students, and Guest users.

- To deploy a centralized server to host file sharing, DHCP, DNS, and web services for internal resource management.

- To provide full Wi-Fi coverage across all classrooms, offices, and common areas using enterprise-grade access points.

- To enable remote monitoring and administration via SSH access to core network devices.

- To simulate and validate the complete network design using Cisco Packet Tracer for functional accuracy and academic demonstration.

- Scope and Limitations

Scope

This project covers the design and simulation of a hierarchical campus network for Villapagasa National High School, with the *Main Distribution Frame (MDF)* located in the ICT Room. It includes the ICT Room (MDF), Admin Office, Computer Lab, JHS Faculty Room, SHS Faculty Room (IDFs), and the four main buildings (Admin, Library, Grade 8, Grade 9-10, SHS). The topology uses *single-mode fiber* for the backbone between buildings and *Cat 6* cabling for horizontal distribution within each facility. It implements VLAN segmentation (Admin, Faculty, Students, Guest), inter-VLAN routing, DHCP, NAT, and basic security (firewall, ACLs, guest isolation). End-device support includes desktops, laptops, printers, IP phones, and enterprise APs for full wireless coverage. Validation is performed in Cisco Packet Tracer.

Limitations

Despite its thoroughness, this project has several limitations. Budget constraints may force the use of lower-grade hardware instead of true enterprise equipment. Internet speed and reliability will depend on the availability and quality of local ISP services in Bansud, Oriental Mindoro. Lastly, the proposal focuses solely on technical infrastructure and does not cover user training or change management programs essential for long-term system sustainability.

Chapter 2

Physical Network Design

- Site Layout

- Map/floor plan of the school (buildings, classrooms, offices, labs, library, etc.)

- Identification of Network Distribution Points

- Cabling Plan

- Backbone Cabling (Between Buildings)

| Location/Building Name |  | Cable Type | Connection Path | Length |
| --- | --- | --- | --- | --- |
| Start Point | End Point |  |  |  |
| MDF (ICT Room) | SHS Faculty Room | Cat5e | Aerial | 110 Meters |
|  | Accounting Office | Cat5e | Aerial | 110 Meters |
|  | Canteen | Cat5e | Aerial | 110 Meters |
|  | Com. Lab | Cat5e | Aerial | 110 Meters |
|  | Guidance Office | Cat5e | Aerial | 110 Meters |
|  | Band Room | Cat5e | Aerial | 110 Meters |
|  | Admin Office | Cat5e | Aerial | 110 Meters |
|  | Covered Court | Cat5e | Aerial | 110 Meters |
| IDF (SHS Faculty Room) | JHS Faculty Room | Cat5e | Aerial | 110 Meters |
|  | JHS Bldg. | Cat5e | Aerial | 110 Meters |
|  | SHS Bldg. | Cat5e | Aerial | 110 Meters |

- Horizontal Cabling (Within Each Building)

| Location |  |  | Cable Type | Length |
| --- | --- | --- | --- | --- |
| Building | Floor |  |  |  |
|  | Start Point | End Point |  |  |
| CBM | IDF Room (1st Floor) | 2nd Floor | Cat6s | 8 Meters |
|  |  | 3rd Floor | Cat6s | 16 Meters |
|  |  | Faculty Room | Cat6s | 16 Meters |
|  |  | Computer Lab | Cat6s | 12 Meters |

- Network Cabinet/Closet Devices

| Location |  | Connected Device | Type of Cable | Quantity | Length |
| --- | --- | --- | --- | --- | --- |
| Bldg., & Floor | Room |  |  |  |  |
| Band Room | Band Room | Access Point | Cat5e | 1 | 10 meters |
| ICT Room | ICT Room | PC | Cat5e | 2 | 10 meters |
|  |  | Access point | Cat5e | 1 | 10 meters |
| Accounting Office |  | printer | Cat5e | 2 | 10 meters |
|  |  | Access point | Cat5e | 1 | 10 meters |
|  |  | pc | Cat5e | 2 | 10 meters |
|  |  | Access point | Cat5e | 1 | 10 meters |
| Covered Court |  | Access Point | Cat5e | 1 | 10 meters |
| Comlab |  | pc | Cat5e | 3 | 10 meters |
|  |  | Access point | Cat5e | 1 | 10 meters |
| Canteen |  | Pc | Cat5e | 1 | 10 meters |
|  |  | Access point | Cat5e | 1 | 10 meters |
| Guidance |  | pc | Cat5e | 2 | 10 meters |
|  |  | Access point | Cat5e | 1 | 10 meters |
| SHS Faculty |  | pc | Cat5e | 2 | 10 meters |
|  |  | printer | Cat5e | 1 | 10 meters |
|  |  | Access point | Cat5e | 1 | 10 meters |
| JHS Faculty |  | pc | Cat5e | 2 | 10 meters |
|  |  | Printer | Cat5e | 1 | 10 meters |
|  |  | Access point | Cat5e | 1 | 10 meters |
| JHS Bldg | G9 room | Access point | Cat5e | 1 | 10 meters |
|  |  | Smart tv | Cat5e | 1 | 10 meters |
|  | 7 | Smart tv | Cat5e | 1 | 10 meters |
|  | 8 | Smart tv | Cat5e | 1 | 10 meters |
|  | 10 | Smart tv | Cat5e | 1 | 10 meters |
| SHS Bldg | G12 | Access point | Cat5e | 1 | 10 meters |
|  |  | Tv | Cat5e | 1 | 10 meters |
|  | G11 | tv | Cat5e | 1 | 10 meters |
|  |  |  | Cat5e | 1 | 10 meters |

- Network Devices Plan

- Intermediary Network Devices

| Location |  | Device | Specification | Quantity |
| --- | --- | --- | --- | --- |
| Bldg., & Floor | Room |  |  |  |
| ICT Room | ICT Room | Access point | TP-Link | 1 |
|  |  | Switch | TP-Link | 1 |
|  |  | Server | TP-Link | 1 |
| Accounting Office | Accounting Office | Access point | TP-Link | 1 |
| Covered Court | Covered court | Access Point | TP-Link | 1 |
| Comlab | Comlab | switch | TP-Link | 1 |
|  |  | Access point | TP-Link | 1 |
| Canteen | Canteen | Access point | TP-Link | 1 |
| Guidance Office | Guidance Office | Access point | TP-Link | 1 |
| SHS faculty | SHS Faculty | switch | TP-Link | 1 |
|  |  | Access point | TP-Link | 1 |
| JHS faculty | JHS Faculty | switch | TP-Link | 1 |
|  |  | Access point | TP-Link | 1 |
| JHS bldg | G9 room | Access point | TP-Link | 1 |
| SHS Bldg | G12 | Access point | TP-Link | 1 |

- Office/Room End Devices

| Location |  | Device | End User | Specification | Quantity |
| --- | --- | --- | --- | --- | --- |
| Bldg. | Room |  |  |  |  |
| ICT Room | ICT Room | PC | Student | Dell | 2 |
| Accounting Office | Accounting Office | printer | Staff | Epson | 2 |
|  |  | pc | Staff | Dell | 2 |
| Comlab | Comlab | pc | Student | Dell | 3 |
| Canteen | Canteen | Pc | Staff | Dell | 1 |
| Guidance | Guidance Office | pc | Staff | Dell | 2 |
| SHS faculty | SHS Faculty | pc | Staff | Dell | 2 |
|  |  | printer | Staff | Epson | 1 |
| JHS faculty | JHS Faculty | pc | Staff | Dell | 2 |
|  |  | Printer | Staff | Epson | 1 |
| Jhs bldg | G9 room | Smart tv | Staff | LG | 1 |
|  | 7 | Smart tv | Staff | LG | 1 |
|  | 8 | Smart tv | Staff | LG | 1 |
|  | 10 | Smart tv | Staff | LG | 1 |
| SHS Bldg | G12 | Smart tv | Staff | LG | 1 |
|  | G11 | Smart tv | Staff | LG | 1 |

- Bill of Materials